AQUAPANEL® Cement Board Outdoor

Directly-applied double-stud exterior wall with metal framework

Manufacture, delivery and installation of a non-load-bearing double-stud exterior wall with metal framework, planked with a single layer AQUAPANEL® Cement Board Outdoor on the exterior and a single layer of Knauf Gypsum Board on the interior, with finished exterior surface. This is a turnkey solution including connections and supplementary services.

# Specification examples

| Item | Quantity / ME | Description of service | Unit cost | Total cost |
| --- | --- | --- | --- | --- |
|  | m2 m2 m2 m2 m2 m2 RM RM | Wall thickness: _________ cm Wall height: _________ cm Ceiling connection to: _____________________________ Floor connection to: _____________________________ Cavity insulation: _____________________________ Insulation thickness: ≥ 140 mm Volumetric weight: ≥ 60 kg/m2 Corrosion-protected substructure in the form of a double framework of continuous top and bottom connection points with edge profiles, support composed of ____________. Installation of the stud profiles at the system interval of 600/625 mm to take the AQUAPANEL® Cement Board Outdoor panels. The profile sizes together with the fasteners and anchors are to be selected in accordance with the stability analysis. Fringe areas and other areas subjected to increased wind pressure and drag loading require that the distances between the pillars be reduced in accordance with the stability analysis. (Intermediate board option) Install single-layer of Knauf Impregnated Gypsum Board or Knauf Fireboard, 12.5 mm, in the middle of the double-stud framework at the inner side of the outer frame using drywall screws. Apply AQUAPANEL® Tyvek® Water-Resistive Barrier with overlapping joints of at least 10 cm over the whole of the surface. Install Knauf Insulation mineral stone wool, in the form of cavity insulation with flush joints, anti-slip. Install AQUAPANEL® Cement Board Outdoor facing 12.5 mm (cementitious, mineral-based construction board, non-combustible (A1), free of cellulose fibres) using the associated AQUAPANEL® Maxi Screws with 3 - 5 mm joint width. Fill all board joints with AQUAPANEL® Joint Filler – grey and AQUAPANEL® Tape (10 cm). Also fill the screw heads maximum 250 mm screw distance. Coating with exterior finish Supply plaster finishing profile, manufacturer/type ___________ and fit to the end of the façade facing plinth using AQUAPANEL® Exterior Basecoat - white, conforming with the layers. Supply plaster finishing profile, manufacturer/type __________and fit to the end of the lintels of the façade facing using AQUAPANEL® Exterior Basecoat - white, conforming with the layers. |  |  |

AQUAPANEL® Cement Board Outdoor

Directly-applied double-stud exterior wall with metal framework

# Specification examples

| Item | Quantity / ME | Description of service | Unit cost | Total cost |
| --- | --- | --- | --- | --- |
|  | RM Unit m2 m2 m2 m2 m2 | Supply plaster finishing profile, manufacturer/type __________and fit to the corners / edges / reveals of the façade facing using AQUAPANEL® Exterior Basecoat - white, conforming with the layers. Apply AQUAPANEL® Reinforcing Mesh, 50 x 30 cm, diagonally above and beneath window, door and other openings. Apply AQUAPANEL® Exterior Basecoat - white, in accordance with the manufacturer’s guidelines, including proper smooth and fold-free embedding of the AQUAPANEL® Reinforcing Mesh. Priming of the exterior basecoat using AQUAPANEL® Basecoat Primer. Proper application and structuring of the exterior finish. Texture: horizontally rendered / vertically rendered / circular rendered / graded. Colour white / other colour tint ____________. Supply and install damp course, air sealing insulation manufacturer/type ________ as necessary. Fit the interior facing of drywall construction board in accordance with the manufacturer’s recommendations. All work to be carried out in accordance with the manufacturer’s recommendations. |  |  |