AQUAPANEL® Cement Board Indoor (Lighter. Easier. Faster.)

#### Skim coat above the tile level in Q4 quality

# Specification examples

| Item | Quantity / ME | Description of service | Unit cost | Total cost |
| --- | --- | --- | --- | --- |
|  | m² m² m² | Wall lining with cement-bonded building boards, primed all-over, AQUAPANEL® Q4-finish above the tile level Above the tile level all visible joints should be covered with AQUAPANEL® Q4- Finish, Knauf fibreglass joint covering strips are to be pressed into the skim coat, apply another thin layer of AQUAPANEL® Q4- Finish to the cover strips, remove residues. Cover all screw heads. When dry, remove irregularities on the surface and screw holes with a hand grinder. Apply another layer of AQUAPANEL® Q4-Finish in a width of at least 20cm onto the existing layer in the joint area. Sand off irregularities again when dry. All-over skim coat of AQUAPANEL® Q4-Finish above the tile level. After drying and interim sanding if necessary, apply a further layer of AQUAPANEL® Q4-Finish. After approx. 24 h drying, the surface can be sanded with sanding paper (grain 120). Very smooth surfaces can be achieved, e.g. with an electric orbital sander. Comment: Height Tile level > Height Area smoothed with AQUAPANEL® Q4-Finish Perform all work according to manufacturer's instructions. |  |  |