AQUAPANEL® Cement Board Indoor (Lighter. Easier. Faster.)

#### Single stud wall with double layer panelling

# Specification examples

| Item | Quantity / ME | Description of service | Unit cost | Total cost |
| --- | --- | --- | --- | --- |
|  | m2 m2 m2 | ## Wall lining with cement-bonded building boards Design: Partition wall construction with double layer panelling without fire protection requirements Non-load-bearing inner partition wall DIN 4103-1 as prefab wall, installation range 1 / 2 *, CW50: installation range 1: 4.00 m, installation range 2: 3.50 m; CW75: installation range 1: 6.00 m, installation range 2: 5.00 m; CW100: installation range 1: 7.00 m, installation range 2: 6.50 m * Wall height: ________ m, Wall thickness: _________ mm , no fire protection requirements, Ceiling junction to: reinforced concrete / masonry _________ / Lightweight concrete / other _________ * Floor joint to: reinforced concrete / masonry _________ / Lightweight concrete / other _________ * rigid peripheral connections, Product / system: AQUAPANEL® Cement Board Indoor (Lighter. Easier. Faster) Making the furring made of galvanised sheet steel sections, corrosion-protected, as single post construction of UW sections _________ mm and CW sections _________ mm; make __________________ . Unit spacing of upright sections ≤ 600 mm / _______ mm*. Insulating layer as required of mineral wool DIN EN 13162, Insulating material thickness: _________ mm Volume weight: _________ kg/m³ Thermal conductivity group _________ Make___________________ , caulked fit and safe from slipping. Additional position: A vapour barrier / vapour seal *, make ____________________, is to be fastened in front of the 2nd panelling on the 1st layer of boards depending on the requirements according to the manufacturer's instructions. |  |  |

AQUAPANEL® Cement Board Indoor (Lighter. Easier. Faster.)

#### Single stud wall with double layer panelling

# Specification examples

| Item | Quantity / ME | Description of service | Unit cost | Total cost |
| --- | --- | --- | --- | --- |
|  | m2 m2 m² linear m m² m² m² m² | Installation of the two-sided panelling of 1st layer AQUAPANEL® Cement Board Indoor (Lighter.Easier.Faster) 12.5 mm, firm butt joints glued with AQUAPANEL® Joint Adhesive(PU) and screwed in place with AQUAPANEL® Maxi Screws SN 25. After hardening (normally the next day) the protruding AQUAPANEL® Joint Adhesive (PU) is scraped off. 2nd layer AQUAPANEL® Cement Board Indoor (Lighter.Easier.Faster) 12.5 mm, firm butt joints glued with AQUAPANEL® Joint Adhesive (PU) and screwed in place with AQUAPANEL® Maxi Screws SN 39. After hardening (normally the next day) the protruding AQUAPANEL® Joint Adhesive (PU) is scraped off. With boards cut to size observe screw spacing of 15 mm to the edge of the board and ≤ 250 mm between screws. Alternative item: 1st layer AQUAPANEL® Cement Board Indoor (Lighter.Easier.Faster) 12.5 mm, 3 – 5 mm joint width and screwed in place with AQUAPANEL® Maxi Screws SN 25. Fill all joints with AQUAPANEL® Joint Filler & Skim Coating – white and AQUAPANEL® Tape (10cm). 2nd layer AQUAPANEL® Cement Board Indoor (Lighter.Easier.Faster) 12.5 mm, 3 – 5 mm joint width and screwed in place with AQUAPANEL® Maxi Screws SN 39. With boards cut to size observe screw spacing of 15 mm to the edge of the board and ≤ 250 mm between screws. Priming the AQUAPANEL® Cement Boards Prime the entire surface of the AQUAPANEL® Cement Board Indoor (Lighter.Easier.Faster) with AQUAPANEL® Board Primer. Edge seal Seal all wall / floor joint areas and wall corners and if necessary all pipe leadthroughs with sealing tapes / gaskets, make ____________________. Full area seal depending on requirements Apply an alternative seal all over, e.g. Knauf all-over seal or _________________* Surface coating Lay the ceramics (tile size ≤ 600x600 mm) with a flexible adhesive, at least C2 acc. to DIN EN 12004. Alternative item: Apply a paintable, all-over surface of AQUAPANEL® Joint Filler and Skim Coating - white in at least 4 mm thickness, smoothed and with an embedded fabric inlay over the entire area near the surface of AQUAPANEL® Reinforcing Mesh. Quality level acc. to leaflet "Indoor plaster surfaces", Deutscher Stuckgewerbebund 2003:Q1 / Q2 / Q3 * screeded / smoothed * Additional position: Apply AQUAPANEL® Q4 Finish acc. to instructions for use on the complete finished surface. Quality level acc. to leaflet "Indoor plaster surfaces", Deutscher Stuckgewerbebund 2003: Q4 |  |  |

AQUAPANEL® Cement Board Indoor (Lighter. Easier. Faster.)

#### Single stud wall with double layer panelling

# Specification examples

| Item | Quantity / ME | Description of service | Unit cost | Total cost |
| --- | --- | --- | --- | --- |
|  | m² | Alternative item: Lay the ceramics (tile size ≤ 600x600 mm), glued with a flexible adhesive, at least C2 acc. to DIN EN 12004, tile level height _____m, above the tile level apply AQUAPANEL® Q4 Finish acc. to processing instructions all-over on the finished surface. Quality level acc. to leaflet "Indoor plaster surfaces", Deutscher Stuckgewerbebund 2003: Q4 Perform all work according to manufacturer's instructions. |  |  |

* Delete where not applicable