AQUAPANEL® Cement Board Outdoor

Ventilated exterior wall with timber framework (+ Refurbishment with installation)

#### Manufacture, delivery and installation of a ventilated exterior wall with timber framework, planked with a single layer

#### of AQUAPANEL® Cement Board Outdoor with finished exterior surface. This is a turnkey solution including connections and supplementary services.

# Specification examples

| Item | Quantity / ME | Description of service | Unit cost | Total cost |
| --- | --- | --- | --- | --- |
|  | m2 RM RM RM m2 m2 RM RM | Supply wooden furring for the AQUAPANEL® Cement Board Outdoor Façade Facing and install such that it is vertically and horizontally aligned using approved anchors and anchor to the existing _____________ substrate. All timber to be at least grade S10 in accordance with DIN 4074/ 24 in accordance with ES 338, impregnated in accordance with DIN 68 800 against fungal disease, decay and insect infestation. Horizontal basic framework, cross-section _______ / _______ mm in accordance with the structural analysis, distance between centres _______ cm in accordance with the structural analysis. Anchor to the substrate using approved plugs and screws manufacturer / type ________ in accordance with the structural analysis. Vertical support framework, cross-section _______ / _______ mm in accordance with the structural analysis, distance between centres 600/625 mm, fasten to the basic framework using approved connectors manufacturer / type ________ in accordance with the structural analysis. The distance between the wall and the front edge of the support framework is _______mm. Supply wooden furring as described under item _____ and fasten to the façade framework in the areas around the window and door reveals and lintels using suitable connectors. Basic framework _______ / _______ mm / mm and support framework _______ / _______ mm / mm. Reveal/lintel widths _____cm. Supply thermal insulation material, e.g. Knauf Insulation, suitable for rear ventilated façade facing, manufacturer / type ________, thickness ____ mm, building material class in accordance with DIN 4102 ______, and fasten to the wall surface with flush joints using appropriate insulation material fasteners recommended by the manufacturer, ______ pcs./m2. Supply thermal insulation material as described under item _____ and fit in the areas around the window and door reveals and lintels. Deviating insulation material thickness ____ mm. Reveal/lintel widths _____cm. Supply and fit aluminium, stainless steel or plastic ventilation profiles, manufacturer / type __________________, colour _________, to the end of the façade furring plinth. Supply and fit exterior windowsills of ________________ manufacturer / type _________________, width ______ mm, colour _________, wide lateral connection pieces including the furring. The length shall be such that the lateral connection pieces are positioned behind the reveal facing. |  |  |

AQUAPANEL® Cement Board Outdoor

Ventilated exterior wall with timber framework (+ Refurbishment with installation)

# Specification examples

| Item | Quantity / ME | Description of service | Unit cost | Total cost |
| --- | --- | --- | --- | --- |
|  | m2 RM RM RM Unit RM m2 m2 | Supply the facing of AQUAPANEL® Cement Board Outdoor 12.5 mm (cementitious, mineral-based construction board, non-combustible (A1), free of cellulose fibres), fit with 3 - 5 mm joint widths and fasten using AQUAPANEL® Façade Screws SN 40 (maximum 250 mm screw distance). Fill all board joints with AQUAPANEL® Joint Filler – grey and AQUAPANEL® Tape (10 cm). Also fill the screw heads. Fasten the reveal and lintel facing of AQUAPANEL® Cement Board Outdoor to the furring as described under item _____. The connections to the window and doorframes are to be constructed with finishing profiles of manufacturer / type __________________, whilst maintaining an expansion joint with a width of ___ mm. The joint is to be closed using pre-compressed sealing tape of manufacturer / type _____________. See below for coating with exterior finish. Coating with exterior finish Supply plaster finishing profile, manufacturer/type ___________ and fit to the end of the façade facing plinth using AQUAPANEL® Exterior Basecoat - white, conforming with the layers. Supply plaster finishing profile, manufacturer/type __________and fit to the end of the lintels of the façade facing using AQUAPANEL® Exterior Basecoat - white, conforming with the layers. Supply plaster finishing profile, manufacturer/type __________and fit to the corners / edges / reveals of the façade facing using AQUAPANEL® Exterior Basecoat - white, conforming with the layers. Apply AQUAPANEL® Reinforcing Mesh, 50 x 30 cm, diagonally above and beneath window, door and other openings. Apply AQUAPANEL® Exterior Basecoat - white, in accordance with the manufacturer’s guidelines, including proper smooth and fold-free embedding of the AQUAPANEL® Reinforcing Mesh. Priming of the exterior basecoat using AQUAPANEL® Basecoat Primer. Proper application and texturing of the exterior finish. Texture: horizontally rendered / vertically rendered / circular rendered / graded. Colour white / other colour tint ____________. All work to be carried out in accordance with the manufacturer’s recommendations. |  |  |