AQUAPANEL® Cement Board Outdoor

Ventilated exterior wall with aluminium framework (+ Refurbishment)

#### Manufacture, delivery and installation of a ventilated exterior wall with aluminium framework, planked with a single layer of AQUAPANEL® Cement Board Outdoor with finished exterior surface. This is a turnkey solution including connections and supplementary services.

# Specification examples

| Item | Quantity / ME | Description of service | Unit cost | Total cost |
| --- | --- | --- | --- | --- |
|  | RM RM m2 RM RM RM Unit RM | Supply an approved metal façade profile in accordance with the structural analysis, type__________/ manufacturer___________ for the AQUAPANEL® Cement Board Outdoor and install in accordance with the manufacturer’s recommendations such that it is vertically and horizontally aligned using suitable anchors and anchor to the existing _____________ substrate. Fasten to the substrate using approved anchors of manufacturer / type__________ in accordance with the structural analysis. Distance between the profile centres 600/625 mm. The distance between the wall and the front edge of the façade profile is _____ mm. The furring is to be constructed in accordance with the manufacturer’s guidelines at the interior and exterior corners of the building. Supply aluminium façade profile as described under item _____ and fit to the areas around the window and door reveals and lintels in addition. The distance between the wall and the front edge of the façade profile is _____ mm. Reveal/lintel widths _____cm. Install AQUAPANEL® Cement Board Outdoor 12.5 mm cementitious, mineral-based construction board, non-combustible (A1), free of cellulose fibres) using the associated AQUAPANEL® Rustproofed Screws with 3 - 5 mm joint width. Fill all board joints with AQUAPANEL® Joint Filler - grey and AQUAPANEL® Tape (10 cm). Also fill the screw heads (maximum 250 mm screw distance). Coating with AQUAPANEL® render. Supply plaster finishing profile, manufacturer/type ___________ and fit to the end of the façade facing plinth using AQUAPANEL® Exterior Basecoat - white, conforming with the layers. Supply plaster finishing profile, manufacturer/type __________and fit to the end of the lintels of the façade facing using AQUAPANEL® Exterior Basecoat - white, conforming with the layers. Supply plaster finishing profile, manufacturer/type __________and fit to the corners / edges / reveals of the façade facing using AQUAPANEL® Exterior Basecoat - white, conforming with the layers. Apply AQUAPANEL® Reinforcing Mesh, 50 x 30 cm, diagonally above and beneath window, door and other openings. Apply AQUAPANEL® Exterior Basecoat - white, in accordance with the manufacturer’s guidelines, including proper smooth and fold-free embedding of the AQUAPANEL® Reinforcing Mesh. |  |  |

AQUAPANEL® Cement Board Outdoor

Ventilated exterior wall with aluminium framework (+ Refurbishment)

# Specification examples

| Position | Quantity | Service description | Unit price | Total price |
| --- | --- | --- | --- | --- |
|  | m2 m2 | Priming of the exterior basecoat using AQUAPANEL® Basecoat Primer. Proper application and texturing of the exterior finish. Texture: horizontally rendered / vertically rendered / circular rendered / graded. Colour white / other colour tint ____________. All work to be carried out in accordance with the manufacturer’s recommendations. |  |  |